Fall Buzzer

Output:

https://drive.google.com/file/d/1TGKUiXcX59j2QRwtlMNDjUA2LTG_jsXF/view?usp=sharing

To make active buzzer beep with a PNP transistor through raspberry pi,

- Connect a buzzer with a positive pin  connected to 3.3V and negative pin connected to the emitter of the transistor while the base is connected to GPIO17 through a 1K resistor and collector is connected to the ground of GPIO.

- Now give power supply to Raspberry pi.

- Connect the pi through VNC viewer.

- Now open python and open a new file.

- Write the code to make Buzzer beep and save it with .py extension(Buzzer.py)

Code:

import RPi.GPIO as GPIO           	# include python lib in RPi

import time

BeepPin = 11	# pin11

def setup():

GPIO.setmode(GPIO.BOARD)    	# Numbers GPIOs by physical location

GPIO.setup(BeepPin, GPIO.OUT)   # Set BeepPin's mode is output

GPIO.output(BeepPin, GPIO.HIGH) # Set BeepPin high(+3.3V) to off beep

def loop():

while True:

GPIO.output(BeepPin, GPIO.LOW)    	# Switch on Buzzer

time.sleep(0.1)    	           	# 0.1s delay

GPIO.output(BeepPin, GPIO.HIGH)

time.sleep(0.1)

def destroy():

GPIO.output(BeepPin, GPIO.HIGH)	# beep off

GPIO.cleanup()                 	# Release resource

print('Press Ctrl+C to end the program...')

setup()

try:

loop()

except KeyboardInterrupt:  # When 'Ctrl+C' is pressed, the child program destroy() will be  executed.

destroy()

- Now to make the Buzzer beep, run the program

- Sudo python Buzzer.py

- Now Buzzer beeps.

- To stop the beep of Buzzer, press ctrl+C