Fall LCD Display

Output: https://photos.app.goo.gl/cCf5DaJx2svQ7ebT6

Components:

- Raspberry Pi

- Breadboard

- LCD 1602

- Potentiometer

- Jumper wires

- 40-Pin GPIO Cable

- T-Extension Board

Connections:

Working:

- Once the wireless connection is established then open thonny Python IDE and open a new file.

- Type in the program and save as LCD.py

# Python Program

from time import sleep

class LCD:

LCD_CLEARDISPLAY    = 0x01    # 0000_0001

LCD_RETURNHOME      = 0x02

LCD_ENTRYMODESET    = 0x04

LCD_DISPLAYCONTROL    = 0x08

LCD_CURSORSHIFT     = 0x10

LCD_FUNCTIONSET     = 0x20

LCD_SETCGRAMADDR    = 0x40

LCD_SETDDRAMADDR    = 0x80

# flags for display entry mode

LCD_ENTRYRIGHT    = 0x00

LCD_ENTRYLEFT     = 0x02

LCD_ENTRYSHIFTINCREMENT   = 0x01

LCD_ENTRYSHIFTDECREMENT   = 0x00

# flags for display on/off control

LCD_DISPLAYON     = 0x04

LCD_DISPLAYOFF    = 0x00

LCD_CURSORON    = 0x02

LCD_CURSOROFF     = 0x00

LCD_BLINKON     = 0x01

LCD_BLINKOFF    = 0x00

# flags for display/cursor shift

LCD_DISPLAYMOVE   = 0x08

LCD_CURSORMOVE  = 0x00

# flags for display/cursor shift

LCD_DISPLAYMOVE   = 0x08

LCD_CURSORMOVE  = 0x00

LCD_MOVERIGHT     = 0x04

LCD_MOVELEFT    = 0x00

# flags for function set

LCD_8BITMODE    = 0x10

LCD_4BITMODE    = 0x00

LCD_2LINE     = 0x08

LCD_1LINE     = 0x00

LCD_5x10DOTS    = 0x04

LCD_5x8DOTS     = 0x00

def __init__(self, pin_rs=27, pin_e=22, pins_db=[25, 24, 23, 18], GPIO = None):

# Emulate the old behavior of using RPi.GPIO if we haven't been given

# an explicit GPIO interface to use if not GPIO:

import RPi.GPIO as GPIO

self.GPIO = GPIO

self.pin_rs = pin_rs

self.pin_e = pin_e

self.pins_db = pins_db

self.used_gpio = self.pins_db[:]

self.used_gpio.append(pin_e)

self.used_gpio.append(pin_rs)

self.GPIO.setwarnings(False)

self.GPIO.setmode(GPIO.BCM)

self.GPIO.setup(self.pin_e, GPIO.OUT)

self.GPIO.setup(self.pin_rs, GPIO.OUT)

for pin in self.pins_db:

self.GPIO.setup(pin, GPIO.OUT)

self.write4bits(0x33) # initialization

self.write4bits(0x32) # initialization

self.write4bits(0x28) # 2 line 5x7 matrix

self.write4bits(0x0C) # turn cursor off 0x0E to enable cursor

self.write4bits(0x06) # shift cursor right

self.displaycontrol = self.LCD_DISPLAYON | self.LCD_CURSOROFF | self.LCD_BLINKOFF

self.displayfunction = self.LCD_4BITMODE | self.LCD_1LINE | self.LCD_5x8DOTS

self.displayfunction |= self.LCD_2LINE

""" Initialize to default text direction (for romance languages) """

self.displaymode =  self.LCD_ENTRYLEFT | self.LCD_ENTRYSHIFTDECREMENT

self.write4bits(self.LCD_ENTRYMODESET | self.displaymode) #  set the entry mode

self.clear()

def begin(self, cols, lines):

if (lines > 1):

self.numlines = lines

self.displayfunction |= self.LCD_2LINE

self.currline = 0

def home(self):

self.write4bits(self.LCD_RETURNHOME) # set cursor position to zero

self.delayMicroseconds(3000) # this command takes a long time!

def clear(self):

self.write4bits(self.LCD_CLEARDISPLAY) # command to clear display

self.delayMicroseconds(3000)  # 3000 microsecond sleep, clearing the display takes a long time

def setCursor(self, col, row):

self.row_offsets = [ 0x00, 0x40, 0x14, 0x54 ]

if ( row > self.numlines ):

row = self.numlines - 1 # we count rows starting w/0

self.write4bits(self.LCD_SETDDRAMADDR | (col + self.row_offsets[row]))

def noDisplay(self):

# Turn the display off (quickly)

self.displaycontrol &= ~self.LCD_DISPLAYON

self.write4bits(self.LCD_DISPLAYCONTROL | self.displaycontrol)

def display(self):

# Turn the display on (quickly)

self.displaycontrol |= self.LCD_DISPLAYON

self.write4bits(self.LCD_DISPLAYCONTROL | self.displaycontrol)

def noCursor(self):

# Turns the underline cursor on/off

self.displaycontrol &= ~self.LCD_CURSORON

self.write4bits(self.LCD_DISPLAYCONTROL | self.displaycontrol)

def cursor(self):

# Cursor On

self.displaycontrol |= self.LCD_CURSORON

self.write4bits(self.LCD_DISPLAYCONTROL | self.displaycontrol)

def noBlink(self):

# Turn on and off the blinking cursor

self.displaycontrol &= ~self.LCD_BLINKON

self.write4bits(self.LCD_DISPLAYCONTROL | self.displaycontrol)

def noBlink(self):

# Turn on and off the blinking cursor

self.displaycontrol &= ~self.LCD_BLINKON

self.write4bits(self.LCD_DISPLAYCONTROL | self.displaycontrol)

def DisplayLeft(self):

# These commands scroll the display without changing the RAM

self.write4bits(self.LCD_CURSORSHIFT | self.LCD_DISPLAYMOVE | self.LCD_MOVELEFT)

def scrollDisplayRight(self):

# These commands scroll the display without changing the RAM

self.write4bits(self.LCD_CURSORSHIFT | self.LCD_DISPLAYMOVE | self.LCD_MOVERIGHT);

def leftToRight(self):

# This is for text that flows Left to Right

self.displaymode |= self.LCD_ENTRYLEFT

self.write4bits(self.LCD_ENTRYMODESET | self.displaymode);

def rightToLeft(self):

# This is for text that flows Right to Left

self.displaymode &= ~self.LCD_ENTRYLEFT

self.write4bits(self.LCD_ENTRYMODESET | self.displaymode)

def autoscroll(self):

# This will 'right justify' text from the cursor

self.displaymode |= self.LCD_ENTRYSHIFTINCREMENT

self.write4bits(self.LCD_ENTRYMODESET | self.displaymode)

def noAutoscroll(self):

# This will 'left justify' text from the cursor

self.displaymode &= ~self.LCD_ENTRYSHIFTINCREMENT

self.write4bits(self.LCD_ENTRYMODESET | self.displaymode)

def write4bits(self, bits, char_mode=False):    # one of functions in class LCD

# Send command to LCD

self.delayMicroseconds(1000) # 1000 microsecond sleep

bits=bin(bits)[2:].zfill(8)

self.GPIO.output(self.pin_rs, char_mode)

for pin in self.pins_db:

self.GPIO.output(pin, False)

for i in range(4):

if bits[i] == "1":

self.GPIO.output(self.pins_db[::-1][i], True)

self.pulseEnable()

for pin in self.pins_db:

self.GPIO.output(pin, False)

for i in range(4,8):

if bits[i] == "1":

self.GPIO.output(self.pins_db[::-1][i-4], True)

self.pulseEnable()

def delayMicroseconds(self, microseconds):

seconds = microseconds / float(1000000) # divide microseconds by 1 million for seconds

sleep(seconds)

def pulseEnable(self):

self.GPIO.output(self.pin_e, False)

self.delayMicroseconds(1)   # 1 microsecond pause - enable pulse must be > 450ns

self.GPIO.output(self.pin_e, True)

self.delayMicroseconds(1)   # 1 microsecond pause - enable pulse must be > 450ns

self.GPIO.output(self.pin_e, False)

self.delayMicroseconds(1)   # commands need > 37us to settle

def message(self, text):

# Send string to LCD. Newline wraps to second line

print ("message: %s",text)

for char in text:

if char == '\n':

self.write4bits(0xC0) # next line

else:

self.write4bits(ord(char),True)

def destroy(self):

print ("clean up used_gpio")

self.GPIO.cleanup(self.used_gpio)

def print_msg():

print ("========================================")

print ("|                LCD1602               |")

print ("|    ------------------------------    |")

print ("|         D4 connect to BCM25          |")

print ("|         D5 connect to BCM24          |")

print ("|         D6 connect to BCM23          |")

print ("|         D7 connect to BCM18          |")

print ("|         RS connect to BCM27          |")

print ("|         CE connect to bcm22          |")

print ("|          RW connect to GND           |")

print ("|                                      |")

print ("|           Control LCD1602            |")

print ("|                                      |")

print ("|                            SunFounder|")

print ("========================================\n")

print ('Program is running...')

print ('Please press Ctrl+C to end the program...')

raw_input ("Press Enter to begin\n")

def main():

global lcd

print_msg()

lcd = LCD()   # obj: lcd

line0 = "  sunfounder.com"

line1 = "---SUNFOUNDER---"

lcd.clear()

lcd.message("Welcome to --->\n  sunfounder.com")

sleep(3)

msg = "%s\n%s" % (line0, line1)

while True:

lcd.begin(0, 2)

lcd.clear()

for i in range(0, len(line0)):

lcd.setCursor(i, 0)

lcd.message(line0[i])

sleep(0.1)

for i in range(0, len(line1)):

lcd.setCursor(i, 1)

lcd.message(line1[i])

sleep(0.1)

sleep(1)

if __name__ == '__main__':

try:

main()

except KeyboardInterrupt:

lcd.clear()

lcd.destroy()

- Run the program in command prompt as:

sudo python LCD.py

- Now you can observe the characters displayed on the LCD.