How to Drive a DC Motor

using RASPBERRY PI 3 GPIO

Output:

https://photos.app.goo.gl/L7e6PjzZoVPz2Nu58

Objective :

The main objective is to drive a DC motor using  the Power Supply Module to supply motors. We achieve this with Python programming using the Raspberry Pi 3 model B development board.

Components:

- Raspberry Pi

- Breadboard

- L293D

- DC Motor

- Power Module

- Jumper wires

Connections:

Working:

Once the wireless connection is established then open thonny Python IDE.

- Type in the program and save as DC_Motor.py

# Python Program

import RPi.GPIO as GPIO

import time

MotorPin1   = 11    # pin11

MotorPin2   = 12    # pin12

MotorEnable = 13	# pin13

def setup():

GPIO.setmode(GPIO.BOARD)      	# Numbers GPIOs by physical location

GPIO.setup(MotorPin1, GPIO.OUT)   # mode --- output

GPIO.setup(MotorPin2, GPIO.OUT)

GPIO.setup(MotorEnable, GPIO.OUT)

GPIO.output(MotorEnable, GPIO.LOW) # motor stop

def loop():

while True:

print 'Press Ctrl+C to end the program...'

GPIO.output(MotorEnable, GPIO.HIGH) # motor driver enable

GPIO.output(MotorPin1, GPIO.HIGH)   # clockwise

GPIO.output(MotorPin2, GPIO.LOW)

time.sleep(5)

GPIO.output(MotorEnable, GPIO.LOW) # motor stop

time.sleep(5)

GPIO.output(MotorEnable, GPIO.HIGH) # motor driver enable

GPIO.output(MotorPin1, GPIO.LOW)   # anticlockwise

GPIO.output(MotorPin2, GPIO.HIGH)

time.sleep(5)

GPIO.output(MotorEnable, GPIO.LOW) # motor stop

time.sleep(5)

def destroy():

GPIO.output(MotorEnable, GPIO.LOW) # motor stop

GPIO.cleanup()                 	# Release resource

setup()

try:

loop()

except KeyboardInterrupt:  # When 'Ctrl+C' is pressed, the child program destroy() will be  executed.

destroy()

- Run the program in command prompt as:

sudo python DC_Motor.py

- Turn on the power module .

- Check the DC motor.