Rotary Encoder

Output: https://photos.app.goo.gl/NNG4r2ttzciwoABg9

Components:

- Raspberry Pi

- Breadboard

- Rotary Encoder Module

- Jumper wires

- 40-Pin GPIO Cable

- T-Extension Board

Connections:

Working:

- Once the wireless connection is established then open thonny Python IDE and open a new file.

- Type in the program and save as encoder.py

# Python Program

import RPi.GPIO as GPIO

import time

RoAPin = 11	# pin11      	-> Connected to CLK

RoBPin = 12	# pin12      	-> Connected to DT

RoSPin = 13	# pin13      	-> Connected to SW

globalCounter = 0

flag = 0

Last_RoB_Status = 0                       	# two var. for pin B’s value

Current_RoB_Status = 0

def setup():

GPIO.setmode(GPIO.BOARD)   	   	# Numbers GPIOs by physical location

GPIO.setup(RoAPin, GPIO.IN)	     	# input mode

GPIO.setup(RoBPin, GPIO.IN)

GPIO.setup(RoSPin,GPIO.IN, pull_up_down=GPIO.PUD_UP)  # Bottom pin

rotaryClear()

def rotaryDeal():

global flag

global Last_RoB_Status

global Current_RoB_Status

global globalCounter

Last_RoB_Status = GPIO.input(RoBPin)        	# Read in data from DT

while(not GPIO.input(RoAPin)):

Current_RoB_Status = GPIO.input(RoBPin)

flag = 1

if flag == 1:

flag = 0

if (Last_RoB_Status == 0) and (Current_RoB_Status == 1):

globalCounter = globalCounter + 1

print 'globalCounter = %d' % globalCounter

if (Last_RoB_Status == 1) and (Current_RoB_Status == 0):

globalCounter = globalCounter - 1

print 'globalCounter = %d' % globalCounter

def clear(ev=None):

globalCounter = 0

print 'globalCounter = %d' % globalCounter

time.sleep(1)

def rotaryClear():

GPIO.add_event_detect(RoSPin, GPIO.FALLING, callback=clear)      	# wait for falling

def loop():

global globalCounter

while True:

rotaryDeal()

#                         	print 'globalCounter = %d' % globalCounter

def destroy():

GPIO.cleanup()         	# Release resource

setup()

try:

loop()

except    KeyboardInterrupt:  # When 'Ctrl+C' is pressed, the child program destroy() will be  executed.

destroy()

- Run the program in command prompt as:

sudo python encoder.py

- Now turn the rotary encoder and you can observe the counter increment or decrement according to the direction of the turn.